Final Reflection: Aaron Haaf

Section 1: Architecture

Designing and deploying a real-time, microsecond-sensitive embedded system, such as an autonomous coaster block-zone controller on a dual-core microcontroller, provides immediate feedback on the trade-offs between initial architectural assumptions and the realities of implementation. Reflecting on the development cycle reveals a few key areas where a fundamental redesign would significantly enhance system reliability and performance: state machine encapsulation, hardware-abstraction isolation, and IPC pipeline selection.

In the initial control firmware, system safety states were tracked using scattered atomic global flags, such as g_brake_engaged and g_estop_latched. While boolean flags offer low performance overhead, they scale poorly as complexity increases. Managing latched fault conditions across asynchronous tasks, such as the high-priority responder task or the event-driven interlock coordinator, introduced several race conditions and logical edge cases that took time to decipher and fix. For instance, determining whether a dispatch button press should be interpreted as an invalid launch attempt or a deliberate manual safety reset required polling global pin levels directly within a general coordination loop.

To fix this in the future, I will replace these discrete global variables with a formal, thread-safe Hierarchical Finite State Machine. By structuring system behavior into distinct parent and child states, state transitions become deterministic and fully verifiable. Entry and exit actions for each state would explicitly manage actuator positions and interlock bitmasks. This change eliminates ambiguous flag combinations and prevents illegal state transitions at the compilation or runtime framework level.

Section 2: Technical Hurdles

Operating within a dual-core symmetric processing environment introduces subtle concurrency challenges. Dividing the software architecture into a Real-Time Plane (Core 1)—running high-priority sensor producers, consumer analysis, and safety responders—and an Observability Plane (Core 0)—handling serial monitoring and web server rendering—introduced cross-core data-sharing hazards.

Race conditions appeared when Core 0 attempted to read global telemetry counters (hb_prod, last_consumed_frame, g_estop_latched) while Core 1 tasks were actively updating them. Without explicit synchronization, Core 0 occasionally read partially written data structures, leading to corrupt serial logs or invalid web dashboard states. Resolving this required strict data access rules: shared scalar variables were declared as volatile with atomic access operations, while complex structures were protected by critical sections or snapshot copies taken within brief mutex locks.

Balancing core workloads also highlighted cross-core priority inversion risks. If a low-priority logging task on Core 0 holds a shared mutex while blocked on a slow UART buffer, a high-priority Core 1 safety task needing that same mutex would stall—temporarily breaking real-time determinism. Recognizing and eliminating these cross-core dependencies required careful inspection of memory boundaries and synchronization primitives.

Section 3: Takeaways

The process of designing, debugging, and refining a real-time safety system yielded technical insights that extend well beyond standard microcontroller programming. The most significant takeaway from this engineering cycle is an appreciation for Fail-Safe Determinism and the Interdisciplinarity of Control Engineering.

In consumer software development, system errors are often handled by retrying operations, swallowing exceptions, or reverting to a default state. In real-time controls and automated safety systems, such as amusement park block-zone enforcement, industrial machinery, or medical devices, this approach is unacceptable. A safety system must treat emergency stops and interlock trips as one-way, latched state transformations. Early iterations of the control code allowed an E-STOP press to act as a simple toggle, enabling subsequent presses or launch requests to open the station brake fin. Re-evaluating this design against real-world safety standards highlighted the danger of that approach: an emergency stop must forcefully drive physical actuators to their safe state (0deg, Brakes Engaged) and latch the system locked. Recovery must require a deliberate, multi-second operator reset procedure. Designing software with a fail-safe mindset requires anticipating hardware failures, stuck pins, sensor dropouts, and invalid state combinations—ensuring that the system defaults to a safe state under every fault scenario.

Conclusion:

The iterative design and implementation of this real-time block-zone safety controller provided a practical demonstration of embedded systems engineering principles. Moving from initial architectural concepts to a stable, dual-core real-time control system highlighted the importance of deterministic state management, rigorous hardware abstraction, and clean IPC separation.

Navigating hardware debouncing constraints, cross-core synchronization risks, and target platform serial configurations reinforced a foundational lesson: reliable control systems are built by designing for the physical realities of hardware execution, anticipating faults at every layer, and prioritizing fail-safe determinism above all else. These insights form a solid framework that will directly inform future engineering work across embedded software development, real-time robotics, and industrial control automation.